The Ultimate POCSAG decoder - MySQL notes

-----------------------------------------

Topics

------

1.0	Installing the client library and headers

1.1	Table columns description and configuration

1.2	Configuration

1.0 Installing the client library and headers

---------------------------------------------

To use MySQL logging with the decoder, you'll need to

have the MySQL client library and header files installed.

On Linux (Debian and Ubuntu) this can be installed with:

apt-get install mysql-client

apt-get install libmysqlclient-dev

On FreeBSD 9.0:

pkg_add -r mysql55-client

1.1 Table columns description

-----------------------------

By default, the program will log to the following table columns:

* date			Date, in the YYYY-MM-DD HH:MM:SS format.

* ric			Pager address.

* ric_uncorrected	If an error is detected in the address codeword,

this column will contain the pre-ECC address,

The column "ric" will contain the post-ECC address.

* bits			Function bits (typically, 0-2 = numerical, 3 = alphanumerical).

* msg			Raw decoded message.

* msg_corrected		Same idea as "ric_uncorrected".

* msg_fixed		Message with character set translation applied (with the exception

of linefeed and carriage return).

* codeword_status	This is a string representing the ECC status of the message

codewords:

0: no error

1: one or two correctable errors

2: uncorrectable error

* codewords		This contains the raw address codeword+message codewords

prepended with the required amount of IDLE codewords to make it

possible to recover the message with the correct RIC.

(MSB byte ordering.)

* codewords_uncorrected	Same idea as "ric_uncorrected".

* bitrate		Integer representing the bitrate the message was decoded at.

* channel		Which channel the message came in on.

In stereo mode, 0 = left, 1 = right.

* errored		0 if the entire message (address codeword+message codewords)

was decoded error-free, 1 otherwise.

1.2 Configuration

-----------------

Run "mysql_config.pl"

The purpose of this utility is to:

* Select which columns you want to log.

* Create the appropriate MySQL database table structure.

* Create a restricted MySQL account that *only* permits

INSERTs into the chosen table.

* Create the corresponding mysql_config.h.

* Display the appropriate db= string.

The contents of mysql_config.h must match the table structure to which you want to log.

If there is a mismatch, you will receive an error message if mysql_config.h

contains references to a column which does not exist in the database table.

If you have the MySQL client library and headers installed, but do not want

to compile the program with MySQL support, edit the "Makefile" and remove

-lmysqlclient and -DUSE_MYSQL (after running "configure").